Name: Rajeshkumar G

Assignment no: 5

1. Pillars of Wi-Fi Security

- Authentication → Verifies identity of user/device

- Confidentiality (Encryption) → Protects data from unauthorized access

- Integrity → Ensures data is not altered during transmission

2. Authentication vs Encryption

- Authentication: Confirms whether a user/device is allowed to connect to the network

- Encryption: Protects the data being transmitted by converting it into a secure format

3. Difference between WEP, WPA, WPA2, WPA3

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Security | Weak | Improved | Strong | Very strong |
| Encryption | RC4 | TKIP | AES | AES + SAE |
| Key Management | Static | Dynamic | Dynamic | Advanced |
| Status | Obsolete | Deprecated | Widely used | Latest |

4. Why WEP is insecure

- Uses weak RC4 encryption

- Uses static keys

- Easily crackable using attacks

- Lacks proper integrity protection

5. Why WPA2 was introduced

- To replace insecure WEP and WPA

- To provide strong encryption using AES

- To improve authentication and data protection

6. Role of PMK in 4-way handshake

- PMK (Pairwise Master Key) is the base key used to derive session keys

- It is used to generate PTK (Pairwise Transient Key) during the handshake

7. How 4-way handshake ensures mutual authentication

- Both client and AP prove they have the same PMK

- Exchange of nonces (ANonce, SNonce)

- Generate PTK independently

- Verify using MIC (Message Integrity Code)

8. What happens if wrong passphrase is used

- PMK generated will be incorrect

- PTK will not match

- MIC verification fails

- Connection is rejected

9. Problem solved by 802.1X

- Provides centralized authentication

- Prevents unauthorized access

- Eliminates shared passwords (like WPA2-PSK)

10. How 802.1X enhances security

- Uses authentication server (RADIUS)

- Provides per-user authentication

- Generates dynamic session keys

- Supports strong authentication methods (EAP)