Name: Rajeshkumar G

Assignment no: 2

1. Split MAC Architecture and its Performance Improvement

In wireless networks, the MAC layer functions are divided between the Access Point (AP) and the Wireless LAN Controller (WLC). This is called Split MAC architecture.

The AP handles time-critical operations such as sending beacons, acknowledging frames, and encrypting data. These tasks need to be done quickly and close to the user device.

On the other hand, the WLC handles higher-level functions like authentication, policy enforcement, and network management.

This separation improves performance because:

- The AP becomes lightweight and efficient

- The network can be centrally managed

- Roaming between APs becomes faster

- The system can easily scale for large networks

2. CAPWAP and Flow between AP and Controller

CAPWAP (Control And Provisioning of Wireless Access Points) is the protocol used for communication between the AP and the controller.

When an AP starts, it first gets an IP address using DHCP. Then it discovers the controller using methods like DNS or DHCP options. After finding the controller, it establishes a secure connection using DTLS.

Once connected, the AP downloads its configuration from the controller and starts working.

From then on:

- Control messages (like configuration updates) go through one channel

- Data traffic (user data) goes through another

This allows centralized control while still handling user traffic efficiently.

3. CAPWAP in OSI Model and its Tunnels

CAPWAP works at the network layer (Layer 3) using UDP.

It uses two types of tunnels:

- The control tunnel is used for sending configuration and management messages between AP and controller. This tunnel is always secured using DTLS.

- The data tunnel is used to carry user traffic. This may or may not be encrypted depending on the setup.

This separation helps in managing the network efficiently.

4. Difference between Lightweight AP and Cloud-based AP

A Lightweight AP depends on a physical controller (WLC) present in the network. All configurations and decisions are made by this controller.

A Cloud-based AP, however, is managed through a cloud platform. There is no need for a physical controller on-site.

In simple terms:

- Lightweight AP → Controlled locally

- Cloud AP → Controlled over the internet

Cloud-based solutions are more flexible and scalable, especially for modern networks.

5. How CAPWAP Tunnel is Maintained

Once the AP connects to the controller, a CAPWAP tunnel is established. This tunnel is maintained using regular keepalive or heartbeat messages.

If the connection is stable, the tunnel remains active. If the connection is lost, the AP will try to reconnect to the controller.

Security is ensured using DTLS encryption, especially for control messages.

6. Sniffer Mode vs Monitor Mode

In sniffer mode, the AP captures wireless packets and sends them to a system for detailed analysis using tools like Wireshark. This is mainly used for troubleshooting and debugging.

In monitor mode, the AP does not serve clients. Instead, it scans the wireless environment to detect interference, rogue access points, and channel usage.

So:

- Sniffer mode → Packet-level analysis

- Monitor mode → Network monitoring

7. Best AP Mode when WLC is in WAN

When the controller is located in a WAN (remote location), the best mode to use is FlexConnect mode.

In this mode, the AP can handle client traffic locally instead of sending everything to the controller. This reduces delay and dependency on WAN connectivity.

Even if the controller becomes unreachable, the AP can still continue basic operations like data forwarding.

This makes FlexConnect ideal for branch offices or remote networks.

8. Challenges with Autonomous APs in Large Networks

Autonomous APs operate independently, without a central controller. While this works for small networks, it becomes difficult in large setups.

Some major challenges include:

- No centralized management

- Difficult to configure and maintain

- Poor scalability

- Inconsistent security settings

- Roaming issues between APs

Because of these reasons, autonomous APs are not suitable for large environments like universities.

9. What Happens if WLC Goes Down (Local Mode AP)

If the controller goes down, the AP loses its connection to it.

In this situation:

- New clients may not be able to connect

- Existing clients may continue for some time

- No new configurations or updates can be applied

If FlexConnect mode is used, the AP can still handle local traffic, which helps maintain connectivity.