Name: Rajeshkumar G

Assignment no: 4

1. Significance of MAC Layer & OSI Position

The MAC (Medium Access Control) layer is part of the Data Link Layer (Layer 2) in the OSI model.

In simple terms, the MAC layer is responsible for deciding how devices use the wireless medium. Since Wi-Fi is a shared medium, multiple devices try to send data at the same time. The MAC layer manages this using techniques like CSMA/CA to avoid collisions.

It also takes care of MAC addressing, frame delivery, and reliability, making sure data reaches the correct device without errors.

So, we can say MAC layer is the controller that manages communication over Wi-Fi.

2. 802.11 MAC Frame Format

The MAC frame is the structure in which data is transmitted over Wi-Fi. It contains different fields, each with a specific purpose.

- Frame Control → tells the type of frame (data, control, or management)

- Duration → specifies how long the channel will be occupied

- Address fields → identify sender, receiver, and destination

- Sequence Control → helps in ordering frames

- Payload → actual data being transmitted

- FCS (Frame Check Sequence) → used for error detection

Overall, the frame format ensures that data is organized, delivered correctly, and checked for errors.

3. MAC Layer Functionalities

The MAC layer performs different functions which can be grouped into three categories:

- Management functions
These include scanning for networks, authentication, and association with access points.

- Control functions
These help manage communication using frames like RTS, CTS, and ACK to avoid collisions.

- Data functions
These handle actual data transmission and features like aggregation and QoS.

So basically, MAC layer handles connection setup, control, and data transfer.

4. Scanning Process and Types

Scanning is how a device finds available Wi-Fi networks.

There are two types:

- Passive Scanning
The device listens for beacon signals sent by access points. It is slower but saves power.

- Active Scanning
The device sends a probe request and AP responds. It is faster but consumes more power.

In real life, devices mostly use active scanning for faster connection.

5. Client Association Process

To connect to a Wi-Fi network, a device follows these steps:

- Scans for available networks

- Selects a suitable access point

- Performs authentication

- Sends association request

- Receives association response

- Gets IP address

After this process, the device is successfully connected to Wi-Fi.

6. EAPOL 4-Way Handshake

This process is used in secure networks like WPA2/WPA3 to establish encryption keys.

Steps:

- AP sends a random value (ANonce)

- Client responds with its own value (SNonce)

- AP sends group key (GTK)

- Client confirms

Two important keys are created:

- PTK → used for secure communication between AP and client

- GTK → used for broadcast traffic

This ensures that communication is secure and encrypted.

7. Power Saving in MAC Layer

Wi-Fi devices use power-saving techniques to reduce battery consumption.

The device goes into sleep mode, and the access point stores data for it. The device wakes up periodically to receive the data.

Types include:

- Legacy Power Save

- U-APSD

- TWT (used in Wi-Fi 6)

This helps in saving battery, especially in mobile devices.

8. Medium Access Control Method

Wi-Fi uses CSMA/CA (Collision Avoidance).

Instead of sending data immediately, a device:

- Checks if the channel is free

- Waits if it is busy

- Uses a random backoff time

- Then transmits

This reduces the chances of collision in wireless networks.

9. Block ACK Mechanism

Instead of sending an acknowledgment for every single frame, Wi-Fi uses Block ACK.

- Multiple frames are sent together

- One acknowledgment confirms all

This reduces overhead and improves performance.

10. A-MSDU, A-MPDU and A-MSDU in A-MPDU

These are techniques used to improve efficiency:

- A-MSDU → combines multiple data packets into one frame

- A-MPDU → combines multiple frames together

- A-MSDU in A-MPDU → combination of both for maximum efficiency

These techniques help in increasing speed and reducing overhead.